Mettler-Toledo AG,Process Analytics

M400 2-wire G2 HART Release Notes

Table of Contents

1	HART Device List	3

1.1	AMS,DeltaV list	3

Note:only AMS v12.0 is tested and is extend to other versions with list above.	3

1.2	PDM list	3

2	Revision History	4

3	Installation for different host	5

3.1	AMS,DeltaV	5

3.2	PDM	5

3.3	Other host	5

4	Know issues and limitations	6

4.1	General	6

4.2	PDM	6

4.2.1	Manufacturer independent problem	6

# HART Device List

## AMS,DeltaV list

| Type | Describtion | Dev Rev | DD Rev | Min AMS version | Min DeltaV version | Firmware |
| --- | --- | --- | --- | --- | --- | --- |
| 0x8e71 | M400 2-wire G2 | 1 | 01 | 10.3 | 10.5 | V_1.0.01 |

### Note:only AMS v12.0 is tested and is extend to other versions with list above.

## PDM list

| Type | Describtion | Dev Rev | DD Rev | Min Required version | Firmware |
| --- | --- | --- | --- | --- | --- |
| 0x8e71 | M400 2-wire G2 | 1 | 01 | 6.0 SP5 | V_1.0.01 |

# Revision History

| Version | Description of changes |
| --- | --- |
| V1.00.00 2023/May/17 | Initial Version. |

# Installation for different host

This chapter describes the standalone installation for different hosts

For AMS software questions, contact the EMERSON Support or refer to the Manual

For further questions, contact the Mettler-Toledo Support.

## AMS, DeltaV

- Use the DD under “DD for AMS_DeltaV”.

- Close all of AMS application. Use the DD installation utility “Add Device Type”to install the DDs. For the detail steps about DD installation, refer to the AMS related installation manual delivered by Emerson.

- If the version of AMS is not listed in ddinstal.ini file, please modified "AmsVersionSupport=10.5,11.0,11.1,11.5,12.0, 14.1, 14,5, xx.x",  xx.x is the user’s AMS version. Another solution is to use “Other host, FM8” files

## PDM

- Please close all applications before the installation start

## Other host

- Use FM8, FMA, or FDI package, refer to the host related installation manual

# Know issues and limitations

## General

- We recommend performing an “Reload Configuration” after change sensor type or channel. Eg: change pH to O2 sensor.

## PDM

### Manufacturer independent problem

- After “Upload to PC/PG”, the display for some of parameters in review menu is incorrect.